Unveiling Waves-Typhoon Amplified Uncertainty of Offshore Wind Power Beyond Speed and its Propagation into Energy Systems Resilience

Zhijia Zhu, Haixin Wang, Member, IEEE, Bicheng Chen, Xinle Hu, Junyou Yang, Mingchao Xia,

Zhe Chen, Fellow, IEEE, Hassan Bevrani, Fellow, IEEE, Rui Jing

This Supplementary provides details for:

S.1 Select the best-fit Copula functions to evidence and describe the wind–wave correlation;

S.2 Simulate Typhoon–Wave interactions by Holland, SWAN, and their coupled model;

S.3 Capture the wave-induced wind speed variability by Wave-Driven Wind Model (WDWM);

S.4 The Scenario settings in the article;

S.5 Quantify wind uncertainty using an ambiguity uncertainty set based on Wasserstein distance;

S.6 Model of Distributionally Robust Optimization Multi-Energy System (DRO-MES).

S.7 Variables used in the Supplementary.

S.1 Select the best-fit Copula functions

This section evidences the correlation exist for wind and wave and explains why we choose t-Copula to describe the wind–wave correlation.

In this study, hourly wind speed and wave height data over three years for the offshore wind farm in Guangdong Province, China, are obtained. Based on these data, the probability density functions (PDF) for wind speed and wave height are first estimated separately, by applying the kernel density estimation (KDE) method. Let (x1,x2,…,xM) be the sample data of the variable X, Let the sample size be M. corresponding to a 24-hour observation period. The PDF is defined in Eq. (s1).

|  |  | (s1) |
| --- | --- | --- |

where K() is the kernel function. In this study, the Gaussian kernel is adopted, i.e., K(x)=12πe-x22; h is the bandwidth parameter that controls the smoothness of the curve. The value of h is calculated using the empirical formula in Eq. (s2).

|  |  | (s2) |
| --- | --- | --- |

where σ is the standard deviation of the sample.

Based on the PDF, the corresponding cumulative distribution function (CDF) can be obtained by integrating the PDF. In practice, an empirical cumulative distribution function (ECDF) is constructed from the sample data to approximate this integral.

The variable X follows a probability density function g(x). If the order statistics satisfy (x1≤x2 ≤ … ≤xM ) then the ECDF GM(x)is defined as Eq. (s3):

|  |  | (s3) |
| --- | --- | --- |

where k is the index of the sample data after sorting, and its value ranges from 1 to M−1.

Subsequently, the ECDF is transformed into a marginal distribution function. The marginal distribution function describes the distribution of a single variable without considering the influence of other variables. For a bivariate variable (X,Y), the marginal distribution function of X is denoted as FX(x)=P(xi≤x), and that of Y is denoted as FY(y)=P(yi≤y).

Then, we use the Copula functions to combine the marginal distribution functions of multiple variables to construct their joint distribution, thereby characterizing the correlation among variables. Commonly used Copula functions include the Gumbel Copula, Clayton Copula, Frank Copula, Gaussian Copula, and t-Copula. To identify the best fit Copula function, the Kendall's rank (τ) and Spearman's rank (ρs) correlation coefficients, along with the Euclidean distances of various Copula functions are calculated and further compared with the empirical values obtained from the sample data.

The formulas for calculating Kendall’s τ and Spearman’s ρs are presented in Eq. (s4) and (s5), respectively. A positive τ indicates a positive correlation between the variables. Spearman’s ρs measures the strength of the monotonic correlation between variables, with higher values indicating stronger monotonic correlation. In this study, we assume that S is wind speed and H is wave height. An array (S,H) is constructed with a series of pairs.

|  |  | (s4) |
| --- | --- | --- |
|  |  | (s5) |

where a is the number of sample pairs exhibiting the same trend of variation, b represents the number of sample pairs exhibiting opposite trends, M is the total number of sampling points, ci represents the rank of the wind speed sample (S1,S2,…,SM) and di represents the rank of the wave height sample (d1, d2,…, dM), and  are the mean rank values, respectively.

The Euclidean distances of various Copula functions are calculated as defined in Eq. (s6). A smaller Euclidean distance indicates a better fit of the corresponding Copula function.

|  |  | (s6) |
| --- | --- | --- |

where si∗ and hi*are the standardized sample values.

The performance of different Copula functions are fitted and presented in Table s1. A stronger correlation is indicated by correlation coefficients closer to 1 and Euclidean distances closer to 0.

Table s1 The coefficients and squared Euclidean distances of various Copula functions

| Copula Type | Euclidean Distance | Kendall | Spearman |
| --- | --- | --- | --- |
| Sample Data | 0 | 0.54093 | 0.73377 |
| Gaussian | 0.053434 | 1 | 1 |
| t-copula | 0.052875 | 1 | 1 |
| Gumbel | 0.24747 | 0.50947 | 0.69318 |
| Clayton | 0.85728 | 0.40312 | 0.56786 |
| Frank | 0.13755 | 0.53693 | 0.73602 |

As shown in Table s1, the t-copula’s Kendall's and Spearman's rank correlation coefficients both equal to 1, and becomes the best fit copula with the smallest Euclidean distance. As illustrated in Figure s1(c), the t-copula captures the tail correlation between wind speed and wave height, accurately reflecting the physical characteristic that higher wind speeds tend to generate higher waves. Although the Gaussian copula also has good correlation in terms of correlation coefficients, its Euclidean distance is slightly larger than that of the t-copula, indicating slightly lower fitting accuracy. In contrast, the Clayton copula shows the largest deviation from a correlation coefficient of 1 and has the largest Euclidean distance, indicating the poorest fitting performance. The Clayton copula is depicts lower-tail correlation between variables, as shown in Figure s1(d), whereas the correlation between wind speed and wave height is mainly concentrated in the upper tail (i.e., extremely high wind speeds are often accompanied by extremely high waves).

Therefore, the t-copula was adopted in this study to characterize the correlation between wind speed and wave height. The joint CDF of the t-copula is given in Eq. (s7).

|  |  | (s7) |
| --- | --- | --- |

where u1 and u2 represent the marginal distribution values of wind speed and wave height, respectively; Vt is the degrees of freedom parameter, with smaller values indicating heavier tails; ∑ and is the covariance matrix that describes the correlation between variables.

Figure s1 Comparison for t-Copula and Clayton -Copula functions. a t-copula joint CDF, b Clayton -copula joint CDF c t-copula joint PDF d Clayton -copula joint PDF

S.2 Holland-SWAN Coupled model for Typhoon–Wave simulation

The theoretical basis for wind–wave coupling is briefed in S.2.1 and Holland-SWAN Coupled model is described in S.2.2.

S.2.1 Mechanisms of wind–wave coupling

As the critical transitional zone between the ocean and atmosphere, the air–sea interface exhibits multi-scale coupled dynamic processes. This interface links the atmospheric boundary layer (ABL) with the marine boundary layer (MBL) and is in a state of dynamic energy equilibrium constantly. When wind blows over the sea surface, it initially induces small perturbations through friction. As the wind speed increases and persists, energy is gradually transferred into the wave field, leading to continuous wave growth. This process is primarily mediated by wind stress, which is often quantified through the drag coefficient, as expressed in Eq. (s8):

|  |  | (s8) |
| --- | --- | --- |

where τs is the wind stress, representing the tangential force per unit area exerted on the sea surface; Cd is the drag coefficient, ρair is the air density, and U10 is the wind speed at 10 meters height (in m/s).

According to the [1], [2], the wind stress τs can be decomposed into the viscous component τvis, the turbulent-associated component τturb, and the component related to air movement caused by waves τwave, as expressed in Eq. (s9).

|  |  | (s9) |
| --- | --- | --- |

where τwave>0 indicates that the wave absorbs energy from the wind; conversely, τwave<0 implies the wave transfers energy back to the wind, thereby accelerating the wind speed.

The magnitude of wind stress directly affects the energy exchange between waves and wind speed, while the three parameters of wave age, significant wave height (one-third of the highest waves), and swell propagation direction jointly determine the magnitude and direction of wave-coherent wind stress, thereby influencing the magnitude and direction of total wind stress. Under swell-dominated conditions, when the wave age is large, the significant wave height is high, and the swells are aligned with the wind direction, the wave-coherent wind stress may oppose the wind direction, thereby reducing the total wind stress. When the swells are opposite to the wind direction, the wave-coherent wind stress aligns with the wind direction, thereby enhancing the total wind stress.

The wave age, typically denoted as U is defined as the ratio between the waves phase speed and the wind speed as shown in Eq (s10). Wave age reflects the maturity of the waves. A higher wave age typically indicates that the waves have developed more maturely, during which more energy is transferred back to the wind, potentially increasing wind speed. In contrast, a lower wave age indicates that the waves are relatively young and in their growth phase, requiring energy absorption from the wind, which is more likely to result in a decrease in wind speed.

|  |  | (s10) |
| --- | --- | --- |

where U represents the wind speed; θp is the angle between the wave propagation direction and the wind direction; and Cp is the wave phase speed. For more detailed information on wind stress and wave age, please refer to the literature [3], [4].

Wu et al. [5] , using the Large-Eddy Simulation (LES) confirmed that the presence of swells significantly alters the wind speed profile, i.e., a phenomenon referred to as wave-driven wind, particularly in the lower regions near the sea surface. Compared to conditions without swells, the disturbances caused by swells make the wind speed profile steeper, especially in the crest regions, which further enhances the variation of wind speed with height.

Figure s2(a) shows the correlation between the normalized wind speed at 30 meters [U/U (30)] and the ABL height Z [m] based on measured data. z/λ represents the ratio of the ABL height to the wavelength. The square curve in black represents the wind speed profile under flat land terrain conditions. It can be observed that the wind speed follows a logarithmic relationship with height. The blue triangle curve represents the wind speed, which begins to increase under the influence of swells, in conditions of growing sea and ocean-atmosphere convection. The red circular curve represents the wind speed under low wind conditions (about 1.4 m/s) with strong swells, where the wind speed reaches its maximum near 8 meters. The green triangle curve corresponds to the wind behaviour with weak swells and a wind speed of 4.7 m/s, where the wind speed gradually stabilizes as the height increases. The LES simulation results in Figure s2(b) evidenced the phenomenon [6].

In addition, waves, by influence wind speed, further influence the output power of wind turbines. Therefore, Alsam et al. [7] employed LES combined with actuator-line techniques to investigate the influence of swells on the power output of offshore wind turbines. The results showed that under the same wind speed, the presence of swell significantly enhanced power generation. Specifically, when the wave age was 1.54, power output increased by approximately 3%, and when the wave age reached 2.17, the increase was about 8.4%. This improvement was primarily attributed to the ability of swell to reduce the total wind stress, thereby increasing wind speed, decreasing wind shear, and decreasing turbulence intensity, which led to a higher power efficiency.

Figure s2. a Measured Vertical Wind Profiles: The black squares represent no swells and neutral conditions; blue triangles represent a growing sea under convective conditions; red circles represent swells during low wind speed (about 1.4 m s⁻¹) under near-neutral conditions; green triangles represent swells during moderate wind speed (about 4.7 m s⁻¹) under convective conditions. b LES Simulated Vertical Wind Profiles: The black solid line represents no swells and neutral conditions; blue dashed line represents no swells and convective conditions; red dashed line represents swells and neutral conditions; green dashed line represents swells and convective conditions. For more detailed information please refer to the literature [5], [6].

S.2.2 Holland typhoon model

To capture the typhoon-induced extreme wind, the triggered massive waves, and the complex interaction between wind-wave, we developed a Holland–SWAN Model, among which the Holland typhoon model is firstly detailed below.

The Holland model is based on the principle of gradient wind balance and is used to describe the pressure distribution and wind speed variation within a typhoon. To apply the Holland model for calculating typhoon-induced pressure and wind speed, key input parameters, i.e., the radius of maximum wind (Rmax), maximum sustained wind speed, and central pressure (Pc) are used.

Assuming the typhoon pressure exhibits axial symmetry, with pressure gradually increasing from the center outward, it can be described by the following Eq. (s11).

|  |  | (s11) |
| --- | --- | --- |

where Pn is the ambient pressure outside the typhoon, Rmax represents the radius of maximum wind, and B is the Holland parameter. r is the radial distance from the typhoon center Generally,

Then, the tangential wind speed can be derived based on the gradient wind balance Eq. (s12).

|  |  | (s12) |
| --- | --- | --- |

where 𝜌air is the air density, taken as 1.15 kg/m³, and 𝑓 is the Coriolis parameter (f=2Ωsinφ), which is dependent on the latitude φ. The correlation between typhoon wind speed, the radial distance from the typhoon center, and the typhoon intensity in the Holland model can be expressed by Eq. (s13), also known as Eq. (1) in the main text.

|  |  | (s13) |
| --- | --- | --- |

where V(r) is the wind speed (in m/s) at a radial distance r, Vm is the maximum wind speed at the typhoon center, The parameters d and e are empirical coefficients, typically ranging between 1 and 2.

S.2.3 SWAN model

The Simulating Waves Nearshore (SWAN) model is constructed to calculate the evolution of waves in nearshore and inland waters. In the Cartesian coordinate system, the equation is expressed as Eq. (s14) [8], also known as Eq. (2) in the main text.

|  |  | (s14) |
| --- | --- | --- |

where N is the wave spectrum density (m2/Hz⋅ rad); Cx, Cy, Cγ and Cθ are the propagation speed of wave in physical space (x, y) , relative frequency space γ, and directional space θ (m/s); γ is the relative frequency (Hz), and θ is the wave propagation direction; S is the source term, including wind energy input, and nonlinear wave–wave interactions, which together represent the energy exchange mechanisms within the wave field. The complete formulation is given in Eq. (s15) [9], also known as Eq. (3) in the main text.

|  |  | (s15) |
| --- | --- | --- |

where Sin is the wind energy input, describing the energy transferred to waves through wind forcing, Snl is the nonlinear interactions, respectively. The dissipation terms Sds,w, Sds,b, Sds,br correspond to whitecapping breaking, bottom friction dissipation, and depth-induced breaking, respectively; each accounting for energy loss due to physical wave processes [10]. More details are further elaborated below.

As shown in Eq. (s15), the wind energy input Sin(γ,θ) in the SWAN model can be expressed as Eq (s16).

|  |  | (s16) |
| --- | --- | --- |

where β is the wave growth coefficient, typically ranging from 0.01 to 0.05; u* is the friction velocity (m/s); g is the gravitational acceleration (9.8 m/s²); E(γ,θ) is the wave energy density spectrum, representing the distribution of energy across frequency γ and direction θ. The bottom friction dissipation term Sds,b(γ,θ) is given by Eq. (s17):

|  |  | (s17) |
| --- | --- | --- |

where Cbot is the bottom friction coefficient, typically ranging from 0.001 to 0.1, depending on the seabed roughness. 𝜌water and 𝜌air represent the densities of water and air, respectively. The depth-induced breaking dissipation term Sds,br(γ,θ) is expressed as Eq. (s18):

|  |  | (s18) |
| --- | --- | --- |

where αbr is the dissipation coefficient for depth-induced breaking; The breaking index is represented by, ξbr, and the breaking probability density Qbr is calculated based on wave height H and water depth d. The formulation is as Eq. (s19):

|  |  | (s19) |
| --- | --- | --- |

S.2.4 Holland–SWAN Coupled Model

Based on above, the Holland–SWAN coupled typhoon–wave model was employed to simulate the wave heights generated by Typhoon Chaba (No. 2203) in 2022. Chaba developed over the South China Sea at 08:00 on June 29, 2022, and moved northwestward. After 24 hours, it intensified from a tropical depression into a tropical storm and continued to strengthen. By 23:00 on July 1, it evolved into a typhoon and approached the Greater Bay Area (GBA) with a maximum central wind speed of 38 m/s. On July 2, Typhoon Chaba made landfall in Dianbai District, Maoming City, with a maximum wind speed of 35 m/s (Category 12). Accordingly, this study defined the potentially affected simulation geographical range within the coordinates 107°E–118°E longitude and 12.5°N–24.5°N latitude. The simulation period was set from 00:00 on July 1 to 18:00 on July 3, totaling 66 hours. The typhoon wind field was updated at 6-hour intervals.

The key typhoon parameters required for Holland typhoon model, i.e., maximum wind speed, radius of maximum wind, translation speed, and typhoon center longitude and latitude, were obtained from the China Meteorological Administration [https://typhoon.nmc.cn/web.html]. In this study, these parameters were input into the Holland typhoon model to calculate the wind speed. The simulated wind speed output, shown in Figure s3, is used as input for the SWAN wave model to simulate the wave height under typhoon conditions. Figure s4a shows the spatial distribution of wave height during the typhoon condition. The simulation results show that, under sustained high wind speeds of Chaba, the maximum wave height in the study area reached 10 meters. From Figure s3 and Figure s4a, the spatial distribution of wave height shows a strong correlation with the distribution of typhoon wind speed.

For validation, the historical wave height data for the same study period (00:00 on July 1 to 18:00 on July 3) were obtained from the ERA5 reanalysis dataset provided by the ECMWF (www.ecmwf.int), as shown in Figure s4b. It is validated that Figure s4a and s4b show similar wave height distribution across the studies area. Note that the simulated wave height could be higher in Figure s4a as the Holland–SWAN Coupled Model could better capture the typhoon-wave coupling at a finer temporal granularity (i.e., 30-minute resolution), whereas the wave height data from ERA5 is characterized by hourly resolution.

In summary, under typhoon condition, there is a stronger correlation between wind speed and wave height, the energy exchange between wind and waves becomes more intense, not only induce a higher wave height, but also significantly increases the uncertainty of wind speed.

Figure s3 Simulated wind speeds for Typhoon Chaba from the Holland typhoon model

Figure s4 Typhoon–Wave Coupled Simulation results. a Simulated wave height considering typhoon-wave coupling, b Historical wave height not considering typhoon-wave coupling.

S.3 Wave-Driven Wind Model (WDWM)

We constructed the Wave-Driven Wind Model (WDWM) to capture the wave-induced wind speed variability under varying wave conditions, and incorporates the Charnock parameter to estimate the sea surface roughness length z0. The formulation used for calculating the roughness length is provided in Eq. (s20) [11].

|  |  | (s20) |
| --- | --- | --- |

where u∗ is the friction velocity, and vwater is the kinematic viscosity of water, taken as 1.4×10−5 m2/s. αch is the Charnock parameter. The first term in the formula, αchu*2/g represents the correlation between wind speed and gravity, reflecting the effect of wind speed on sea surface roughness. The second term, 0.11v/u*, accounts for the molecular viscosity of air. It plays a certain corrective role for wind speed variations. It is important to note that the Charnock parameter αch is not a constant, but rather dependent on wave stress (wave-induced momentum flux) [12]. In this study, the ERA5 hourly reanalysis Charnock dataset is used to obtain the necessary input parameters. αch calculation is shown in Eq (s21).

|  |  | (s21) |
| --- | --- | --- |

where, τs is the total surface wind stress and τwave represents the wave stress. The friction velocity can be expressed as Eq (s22):

|  |  | (s22) |
| --- | --- | --- |

The momentum exchange between air and water can be described by the drag coefficient Cd, and the specific relationship is given as Eq.(s23):

|  |  | (s23) |
| --- | --- | --- |

Based on the above formulation, the momentum exchange between air and water can be expressed in terms of the friction velocity u∗, thereby enhancing the accuracy of wind profile predictions. To analyze the variation of wind speed with height, the logarithmic wind profile model is commonly employed to describe wind distribution in the ABL. This model assumes that, under idealized conditions, there exists a logarithmic relationship between wind speed and height z. The corresponding expression is provided in Eq. (s24).

|  |  | (s24) |
| --- | --- | --- |

where, z is the vertical height above the ground surface, 𝜅=0.4 is the von Kármán constant, which can provide a more accurate depiction of wind speed variations due to the interaction between waves and wind. Since the ERA5 raw wind is based on atmospheric reanalysis, relying mainly on atmospheric observations and model simulations, the impact of waves on wind speed is primarily reflected in the friction between the wind and waves. This frictional change can cause fluctuations in wind speed. However, the raw wind does not account for the interaction between waves and wind, and therefore, it does not accurately reflect the impact of waves on wind speed. In contrast, the WDWM considers the interaction between waves and wind, generating wind speed profiles that show a similar distribution to raw wind speeds, but with slightly more intense fluctuations, as shown in Figure s5.

Figure s5 Comparison between WDWM generated and Historical Wind Speed Profiles.

Townsend et al. [13] provided an in-depth analysis of the enhanced wind speed profile caused by wave–wind interactions, and experimentally quantified their influence on wind speed at various heights along a wind turbine blade. These findings demonstrate that wave-induced disturbances can significantly influence wind resource availability at different vertical positions along the rotor plane of wind turbines. Figure s6 illustrates the mean streamwise velocity profiles within the ABL under different wind–wave interaction conditions. This figure compares representative cases with varying wave heights (H) and wavelengths (λ) to highlight the influence of wave-induced effects. The blue dashed line is the incident Computational Fluid Dynamics (CFD) ABL profile, which represents the original inlet wind velocity distribution obtained from the CFD simulation prior to accounting for wave effects. The red dotted line represents the averaged ABL, which indicates the representative wind speed profile obtained by sample averaging under identical conditions, serving as a baseline for comparison. The green line represents the re-calibrated atmospheric boundary layer (Re-calibrated ABL), in which the power exponent (α) is introduced to adjust the vertical gradient of the wind speed profile, and the wind risk factor (kf) is applied to modify the overall wind speed magnitude, thereby providing a more realistic representation of the wind–wave interaction effects on the ABL wind structure. Hence, Figure s6 indicates that when the significant wave height was 0.3 m and the wavelength was 3 m, the wind speeds at the lowest blade nadir, the hub height, and the blade zenith increased by 10.1%, 3.1%, and 1.3%, respectively.

Figure s6 Wind Speed Variation at Different Rotor Heights under Wave–Wind Interaction[13]

Based on the findings from reference [13], the wind speed increases by different percentages under various significant wave heights (0.3 m and 0.03 m) and wavelengths (3 m and 1.61 m).

In this study, numerical results indicate that wave-induced wind speed increments range from ~1.3% to ~10.1% along the wind turbine height. Considering that the blade nadir region is more sensitive to surface wave effects and turbulence, and typically exhibits a relatively low wind speed, the 10.1% increase was chosen for detailed analysis. Based on this increment, the final wind speed under wave-induced is then calculated using the adjustment Eq. (s25).

|  |  | (s25) |
| --- | --- | --- |

where uf(z) represents the wave-driven wind speed, while uac(z) is the measured wind speed at 10 m height, β is set to 10.1%.

S.4 Scenario settings

When setting up scenarios, we considered the influence of typhoon conditions and normal conditions on the MES. We use the Typhoon-Wave Evolution Dataset (TWED), as simulated in section S.2, as input for the typhoon scenes. Meanwhile, the wind speed dataset driven by wave, generated by the WDWM introduced in section S3, covering a time span from 2022 to 2024. This dataset, characterized by a 3-year scale and a broad range of scenarios, is subjected to K-medoids clustering to generate conventional scenarios.

Considering the distinct seasonal variations in wind speeds in GBA, we further improve the clustering process. In specific, during the summer, when there is no typhoon, the influence of tropical high pressure often results in low wind speeds. Before the arrival of a typhoon, wind speeds frequently exceed 10 m/s (medium wind speed) due to the influence of the typhoon. Additionally, in the winter, the wind speeds near the Taiwan Strait exit are typically higher due to the winter monsoon. Therefore, two different wind speed scenarios are clustered: one for low wind speeds in summer, medium wind speeds in summer and winter. The two types of wind speed are shown in Figure s7. Based on wind speed intensity, wave levels, grid connectivity condition, and seasonal load types, a total of six representative scenarios is constructed as listed in Table 1 in the main text.

Figure s7 The two types of wind speed. a. Low wind speed, b. Medium wind speed.

S.5 Wasserstein-based wind uncertainty distribution

In this section, the uncertainty of offshore wind power output is modeled using an ambiguity uncertainty set based on the Wasserstein distance. Specifically, a data-driven ambiguity set for offshore wind power is constructed using the three-year wave-driven wind speed dataset generated in Section S.3. This uncertainty set captures plausible probability distributions under a given confidence level, thereby decreasing the over-conservatism often encountered in traditional robust optimization frameworks.

S.5.1 Wasserstein distance-based wind power uncertainty modeling

The prediction error between the predicted and actual wind power output at instance ξi is denoted as Eq.(s26):

|  |  | (s26) |
| --- | --- | --- |

where Piw and Piw represent the predicted and actual output of offshore wind power, respectively. In this study, the output deviation ξ is a variable. Assuming the probability distribution of ξ is unknown but that a set of M independent and identically distributed historical samples is available, {ξ1,ξ2, …,ξM},the empirical distribution QN can be used to estimate the true distribution P of ξ , as Eq.(s27) [14], [15].

|  |  | (s27) |
| --- | --- | --- |

where, δξi is the Dirac measure, which indicates the presence of sample point ξi in the sample set. Specifically, δξi=1 if the point belongs to the set, and δξi=0 otherwise.

The DRO ambiguity set based on Wasserstein distance is centered on the empirical distribution of the uncertain random variable ξ , using QN as an estimate of the unknown true distribution P. Then, based on QN, an ambiguity set that potentially contains P is constructed, satisfying the convergence condition: limM→∞QN=P, To formalize the closeness between QN and P the Wasserstein distance is introduced. The Wasserstein distance is given as Eq.(s28):

|  |  | (s28) |
| --- | --- | --- |

where ξM and ξ are uncertain parameters drawn from the empirical distribution QN and the true distribution P, respectively. Π represents a joint probability measure with marginals QN and P. The function dξM,ξ=∥ξM-ξ∥ describes the transportation cost between distributions, i.e., the effort required to transfer mass from P to QN. Thus, the Wasserstein distance quantifies the minimum total cost needed to transform one distribution into another [15]. Based on this, the ambiguity set can be defined as Eq.(s29):

|  |  | (s29) |
| --- | --- | --- |

This ambiguity set in Eq.(s29) can be viewed as a Wasserstein ball centered at QN with radius ε(M). The radius reflects the confidence level in approximating the true distribution, and is linked to the sample size M. The radius function is expressed as Eq.(s30) and Eq.(s31):

|  |  | (s30) |
| --- | --- | --- |
|  |  | (s31) |

where, 1-β1 represents the confidence level associated with the radius of the Wasserstein ball; ξti is the mean of all samples; ∂ is parameter; and ξM is the M-th uncertain parameter. It can be observed that as the radius of the Wasserstein ball increases, the ambiguity set A expands, thereby covering a broader range of uncertain distributions [16].

S.5.2 Data-driven support set

In this context, the ambiguity set is defined over a specific domain, where the support set Ξ represents the set of all points in that domain with a non-zero membership grade. According to references[15], [17], the support set Ξ can be constructed using the sample data {ξ1,ξ2,…,ξM}. The sample set is first standardized using normalization as Eq.(s32):

|  |  | (s32) |
| --- | --- | --- |

where ∑ is the sample covariance matrix, and ξi is the i-th sample. After normalization, the uncertain parameter samples θiu have zero mean and unit variance. Let Φ be the support set of θiu, as defined in Eq.(s33):

|  |  | (s33) |
| --- | --- | --- |

where θi is the i-th element in θu. and l represents the boundary of θi. R is the set of real numbers.

Therefore, the goal of constructing the set Φ is to determine a reasonable parameter l, such that on one hand, the event  for the random variable θi occurs with high probability, and on the other hand, the size of the support set Φ is kept as small as possible to enhance the conservativeness of the model. To this end, the optimization problem for l is formulated as Eq.(s34):

|  |  | (s34) |
| --- | --- | --- |

where lmax is the upper bound of l; pstd and Astd represent the probability distribution and ambiguity set of the standardized uncertain parameter θu, respectively. Once the optimal l is determined, the data-driven support set Ξ can be obtained as Eq.(s35):

|  |  | (s35) |
| --- | --- | --- |

S.6 DRO-MES model formulation

The mathematic formulation for the Wasserstein-based Distributionally Robust Optimization Multi-Energy System (DRO-MES) optimization model is explicated here. S.6.1 firstly defined all the sets and variables in the model.

S.6.1 First-stage objective function and constraints

The DRO-MES optimization model’s first-stage objective function and constraints are derived below.

S.6.1.1 First-Stage objective function

The objective function is to minimize the total operating cost (TOC) of WSDRO-MES, which considers the equipment operation cost Cope, electricity purchase cost Cebuy, natural gas purchase cost Cgbuy, and the regulation cost caused by offshore wind power fluctuation supP∈AEP(Q(x,ξ)) as shown by Eq. (s36).

|  |  | (s36) |
| --- | --- | --- |

where Cebuy, Cgbuy, Cope are further derived in Eq. (s37) ~ (s39).

|  |  | (s37) |
| --- | --- | --- |

where λe,tbuy is the unit price of purchasing electricity from the power grid and Pgrid,tbuy is the amount of electricity purchased from the power grid.

|  |  | (s38) |
| --- | --- | --- |

where, λgbuy is the unit price of purchasing gas from the gas network and Pgas,tbuy is gas volume purchased from the gas network.

|  |  | (s39) |
| --- | --- | --- |

where CCHP, CGB, CHP, CAC and CBESS are operation and maintenance costs of the CHP, gas boiler, heat pump, Absorption chiller and battery energy storage system (BESS), respectively.

S.6.1.2 First-stage constraints

Energy balance constraints. Electricity, heating and cooling balances are modeled. The heating balance is presented in Eq. (s40).

|  |  | (s40) |
| --- | --- | --- |

where t is the time interval, and in this study, the dispatch time spans 48 hours with a 15-minute interval. HtCHP is output of the CHP at time t; HtGB denotes the heating output of the gas boiler at time t; Htload is the heating load; HtHP is the heating supply from heat pump, HtAC is the heating consumed by absorption chiller.

Eq. (s41) shows the electricity balance.

|  |  | (s41) |
| --- | --- | --- |

where Ptbuy is the electricity purchased from the power grid; PtPV is the Photovoltaic power generation; Ptw is the wind output; PtCHP is the electricity output of the CHP; Ptcha and Pt dis denote the charging and discharging power at time t; Ptload is the electricity load; PtHP is the electricity consumed by heat pumps.

Eq. (s42) shows the cooling balance.

|  |  | (s42) |
| --- | --- | --- |

where CtHP is the cooling supplied by heat pump, CtAC is the cooling supplied by absorption chiller, Ctload is the cooling load.

Operational constraints. The heat pump, PV, WT, CHP, gas boiler, and absorption chiller energy conversion constraints are shown by Eq. (S43a~g). WT installation areas have a physical limit as constrained in Eq. (S43h).

|  |  | (S43a) |
| --- | --- | --- |
|  |  | (S43b) |
|  |  | (S43c) |
|  |  | (S43d) |
|  |  | (S43e) |
|  |  | (S43f) |
|  |  | (S43g) |
|  |  | (S43h) |

where ηhp,c ηhp,h are the coefficients of performance (COP) in cooling and heating modes, respectively; Poutw denote rated power output of the offshore wind turbine, Vwind denote actual wind speed, Vin and Vout denote cut-in (3m/s) and cut-out wind speeds (25m/s) respectively, ρair is air density, Cp is aerodynamic power coefficient; GtCHP represents the natural gas consumption rate of the CHP at time t; ηchp,e and ηchp,h denote the electrical efficiency and thermal efficiency of the CHP, respectively; λCH4 is the lower heating value (LHV) of natural gas, the lower heating value of natural gas is taken as 9.78 kWh/m³, GtGB represents the natural gas consumption rate of the gas boiler at time t, ηGB,h is the thermal efficiency of the gas boiler; ηtAC is the thermal-to-cooling conversion efficiency; Pwt,max is the maximum wind power output of the wind farm.

Battery storage constraints. Battery storage is modeled as in Eq. (S44a~e).

|  |  | (S44a) |
| --- | --- | --- |
|  |  | (S44b) |
|  |  | (S44c) |
|  |  | (S44d) |
|  |  | (S44e) |

where Pmaxdis and Pmincha are denote the upper bounds of charging and discharging power, respectively; ut is the binary state variable indicating the charging or discharging status of the battery (0 or 1), u is the self-discharge rate of the battery storage system, Emin and Emax represent the minimum and maximum energy limits of the battery storage system, respectively.

Capacity limit constraints. The capacity limits constraints for each device are given by Eq. (45a~h), along with the corresponding capacity and ramp-up constraints, as shown in these equations.

|  |  | (s45a) |
| --- | --- | --- |
|  |  | (s45b) |
|  |  | (s45c) |
|  |  | (s45d) |
|  |  | (s45e) |
|  |  | (s45f) |
|  |  | (s45g) |
|  |  | (s45h) |

where PdownCHP and PupCHP are the ramp-down and ramp-up limits of electric power generation, PminCHP and PmaxCHP are the minimum and maximum power outputs of the CHP; HdownGB and HdownGB denote the ramp-down and ramp-up limits of the gas boiler, respectively; HminGB and HmaxGB represent the lower and upper bounds of the boiler’s heating output; CminHP and CmaxHP represent the lower and upper bounds of the heat pump’s cooling output; HminHP and HmaxHP represent the lower and upper bounds of the heat pump’s heating output; CminAC and and CmaxAC represent the lower and upper bounds of the absorption chiller’s cooling output; CdownAC and CdownAC denote the ramp-down and ramp-up limits of the absorption chiller, respectively.

Grid connection constraints. The electricity imported from the grid is formulated by Eq. (s46).

|  |  | (s46) |
| --- | --- | --- |

where Pt,maxbuy represents the electricity purchase upper limit. No electricity feedback to power grid is setup.

S.6.2 Second-stage objective function and constraints

The DRO-MES model’s second-stage function and constraints are derived below.

S.6.2.1 Second-stage constraints

To characterize the error between the actual offshore wind power output and the predicted output, this study introduces the disturbance variable ξt. This construction not only simplifies the model complexity but also enhances the adaptability of control strategies under uncertain conditions. The specific formulation is described in Eq. (s47):

|  |  | (s47) |
| --- | --- | --- |

where, Ptw denote the actual power outputs of wind generation at time t, while Ptw represent the corresponding predicted values. The disturbance variable ξt is introduced to characterize the aggregate renewable generation shock faced by the system.

Operational adjustment constraints. Eq. (S48a~i) describe the response models of typical flexible resources in the system, including CHP, gas boilers, heat pumps, absorption chillers, battery energy storage systems, and grid-connected electricity exchanges. These devices are modeled based on their energy conversion characteristics and operating constraints. Each device builds an affine relationship with respect to the disturbance variable ξt , either in terms of output rate or energy capacity, thereby enabling dynamic regulation in response to renewable power fluctuations. By assigning individual response coefficients αti to each device, the system can flexibly coordinate the output adjustment capabilities of different energy carriers, achieving robust multi-energy collaborative operation.

|  |  | (s48a) |
| --- | --- | --- |
|  |  | (s48b) |
|  |  | (s48c) |
|  |  | (s48d) |
|  |  | (s48e) |
|  |  | (s48f) |
|  |  | (s48g) |
|  |  | (s48h) |
|  |  | (s48i) |

where Ptd,CHP, Htd,CHP , Htd,GB, Ctd,HP, Htd,HP, Ctd,AC, Ptd,cha, Ptd,dis, Ptd,buy represent the adjustment power for CHP, GB, HP, AC,BESS, and electricity purchase, respectively.

Capacity limit adjustment constraints. Eq. (s49a~h) further introduced to ensure that the system operation remains physically feasible and consistent with the operational characteristics of the equipment. These constraints include upper and lower bounds on power adjustment and ramping dynamics. Such constraints not only ensure that the adjusted output remains within a reasonable range after disturbance correction, but also help enhance the stability of the overall system dispatch.

|  |  | (s49a) |
| --- | --- | --- |
|  |  | (s49b) |
|  |  | (s49c) |
|  |  | (s49d) |
|  |  | (s49e) |
|  |  | (s49f) |
|  |  | (s49g) |
|  |  | (s49h) |

Battery storage adjustment constraints. BESS is adjustment constraints as show Eq.(s50).

|  |  | (s50) |
| --- | --- | --- |

Energy balance adjustment constraints. To address the energy balance fluctuations caused by renewable power output disturbances, Eq. (s51a~c) establish the affine adjustment constraints for electricity, heating, and cooling balances.

|  |  | (s51a) |
| --- | --- | --- |
|  |  | (s51b) |
|  |  | (s51c) |

S.6.2.2 Second-Stage Objective function

In conjunction with the affine adjustment constraints of the second stage, the system dynamically responds through various technologies after a disturbance occurs to maintain multi-energy balance. Eq. (s52) describes the worst-case expected operation cost of the MES after considering the wind output uncertainty.

|  |  | (s52) |
| --- | --- | --- |

where Cgb represents the heating production cost of the gas boiler, and Cstore denotes the operation cost of charging and discharging BESS. Chp, Hhp and Cac, represent the cooling production cost of the heat pump, the heating production cost of the heat pump, and the cooling production cost of the absorption chiller, respectively.

# S.7 Variable definitions

The variables in the supplementary have been listed in Table S2 and S3.

Table S2 Definitions of indices

| Indices/subscript/ superscript | Definitions |
| --- | --- |
| T | Set of 48 hours |
| t | Set of 15-minute intervals within 48 hours |

Table S3 Definitions of variables

| Variables |  | Definitions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TOC |  | The objective of minimize the total operating cost [¥] |  |  |  |  |
| LHV |  | The lower heating value |  |  |  |  |
| DRO-MES |  | Distributionally Robust Optimization Multi-Energy System |  |  |  |  |
| COP BESS CHP HP PV AC GB WT cha/dis ξt αti QN P δξi Ptw Ξ V(r) Vm Cx, Cy, Cγ Cθ Sin Snl Sds,w Sds,b |  | The coefficients of performance Battery Energy Storage Systems Combined Heating and Power Heat Pump Photovoltaic Absorption Chiller Gas Bolier Wind Turbine charge/discharge The disturbance variable The response coefficients to each device of disturbance variable The empirical distribution The true distribution The Dirac measure The predicted wind power outputs at time t The support set The wind speed at a radial distance r The maximum wind speed at the typhoon center The propagation speed of wave energy in physical space (x,y, γ, θ) The wind energy input The nonlinear wave–wave interactions The whitecapping breaking The bottom friction dissipation |  |  |  |  |
| Binary Variables |  |  |  |  |  |  |
| ut |  | =1 if charging, and = 0 if discharging. |  |  |  |  |
| Positive Variables |  |  |  |  |  |  |
| Cope | The equipment operation cost [¥] |  |  | Cebuy | The electricity purchase cost [¥] |  |
| Cgbuy | The gas purchase cost [¥] |  |  | Ptbuy | The electricity bought from the grid [MW] |  |
| λe,tbuy | The electricity price[kWh/¥] |  |  | PtPV | The electricity generated from PV [MW] |  |
| λgbuy | The natural gas price[m3/¥] |  |  | Ptw | The electricity generated from offshore wind turbine [MW] |  |
| ηchp,e | The electrical efficiency of CHP |  |  | PtCHP | The electricity generated from CHP [MW] |  |
| ηchp,h | The heating efficiency of CHP |  |  | Ptcha | The electricity charged into battery [MW] |  |
| ηGB,h | The heating efficiency of gas boiler |  |  | Pt dis | The electricity discharged from battery[MW] |  |
| ηtAC | The cooling efficiency of gas boiler |  |  | Ptload | The electricity load [MW] |  |
| λCH4 | The lower heating value of natural gas |  |  | PtHP | The electricity consumed by heat pump [MW] |  |
| ηBESS | The electrical efficiency of battery storage system |  |  | CtHP | The cooling output from heat pump [MW] |  |
| Pgrid,tbuy | The electricity bought from the grid [kWh] |  |  | CtAC | The cooling output from absorption chiller [MW] |  |
| Pgas,tbuy | The natural gas bought from the grid[m3] |  |  | Ctload | The cooling load [MW] |  |
| CCHP | The operation and maintenance costs of the CHP[¥] |  |  | Poutw | The rated power output of the offshore wind [MW] |  |
| CGB | The operation and maintenance costs of the gas boiler[¥] |  |  | Pwt,max | The maximum wind power output of offshore wind [MW] |  |
| CHP | The operation and maintenance costs of the heating pump[¥] |  |  | GtCHP | The natural gas consumed by CHP [m³] |  |
| CAC | The operation and maintenance costs of the absorption chiller [¥] |  |  | GtGB | The natural gas consumed by gas boiler [m³] |  |
| HtCHP | The heating output from CHP [MW] |  |  | Ptd,CHP | The adjustment electricity generated from CHP [MW] |  |
| HtGB | The heating output from gas boiler [MW] |  |  | Htd,CHP | The adjustment heating generated from CHP [MW] |  |
| Htload | The heating load [MW] |  |  | Htd,GB | The adjustment heating generated from gas boiler [MW] |  |
| HtHP | The heating output from heat pump [MW] |  |  | Ctd,HP | The adjustment cooling generated from heat pump [MW] |  |
| HtAC | The heating consumed by absorption chiller [MW] |  |  | Htd,HP | The adjustment heating output from heat pump [MW] |  |
| Ptd,dis | The adjustment electricity discharged from battery [MW] |  |  | Ctd,AC | The adjustment cooling output from absorption chiller [MW] |  |
| Ptd,buy | The adjustment electricity bought from the grid [MW] |  |  | Ptd,cha | The adjustment electricity charged into battery [MW] |  |

# Supplementary Reference

[1] M. B. Paskyabi, S. Zieger, A. D. Jenkins, A. V. Babanin, and D. Chalikov, “Sea Surface Gravity Wave-wind Interaction in the Marine Atmospheric Boundary Layer,” Energy Procedia, vol. 53, pp.184–192, 2014.

[2] H. Branger, M. A. Manna, C. Luneau, M. Abid, and C. Kharif, “Growth of surface wind-waves in water of finite depth: A laboratory experiment,” Coast. Eng., vol. 177, pp.104174, Oct. 2022. https://doi.org/10.1016/j.coastaleng.2022.104174

[3] D. Myrhaug, “Wind gust spectrum over waves: Effect of wave age,” Ocean Eng., vol. 34, no. 2, pp.353–358, Feb. 2007. https://doi.org/10.1016/j.oceaneng.2006.01.003

[4] C. E. Villarreal-Olavarrieta, F. J. Ocampo-Torres, P. Osuna, and R. E. Mora-Escalante, “Effect of waves on the magnitude and direction of wind stress over the Ocean,” Ocean Model., vol. 191, pp.102420, Oct. 2024. https://doi.org/10.1016/j.ocemod.2024.102420

[5] L. Wu, E. Sahlée, E. Nilsson, and A. Rutgersson, “A review of surface swell waves and their role in air–sea interactions,” Ocean Model, vol. 190, pp.102397, Aug. 2024. https://doi.org/10.1016/j.ocemod.2024.102397

[6] E. O. Nilsson, A. Rutgersson, A. S. Smedman, and P. P. Sullivan, “Convective boundary‐layer structure in the presence of wind‐following swell,” Q. J. R. Meteorol. Soc., vol. 138, no. 667, pp.1476–1489, Jan. 2012. https://doi.org/10.1002/qj.1898

[7] A. AlSam, R. Szasz, and J. Revstedt, “The Influence of Sea Waves on Offshore Wind Turbine Aerodynamics,” J. Energy Resour. Technol., vol. 137, no. 5, pp.209, Jul. 2015. https://doi.org/10.1115/1.4031005

[8] G. Li, H. Zhang, T. Lyu, and H. Zhang, “Regional significant wave height forecast in the East China Sea based on the Self-Attention ConvLSTM with SWAN model,” Ocean Eng., vol. 312, pp. 119064, Nov. 2024. https://doi.org/10.1016/j.oceaneng.2024.119064

[9] N. G. Valiente, A. Saulter, J. M. Edwards, H. W. Lewis, J. M. Castillo Sanchez, D. Bruciaferri, et al., “The Impact of Wave Model Source Terms and Coupling Strategies to Rapidly Developing Waves across the North-West European Shelf during Extreme Events,” J. Mar. Sci. Eng., vol. 9, no. 4, pp.403, 2021. https://doi.org/10.3390/jmse9040403

[10] P. A. E. M. Janssen, “Quasi-linear Theory of Wind-Wave Generation Applied to Wave Forecasting,” J. Phys. Oceanogr., vol. 21, no. 11, pp.1631–1642, Nov. 1991. https://doi.org/10.1175/1520-0485(1991)021<1631:QLTOWW>2.0.CO;2

[11] X. G. Larsén, J. Du, R. Bolaños, M. Imberger, M. C. Kelly, M. Badger, et al., “Estimation of offshore extreme wind from wind‐wave coupled modeling,” Wind Energy (Chichester Engl.), vol. 22, no. 8, pp.1043–1057, Apr. 2019. https://doi.org/10.1002/we.2339Digital Object Identifier (DOI)

[12] N. Makrygianni, S. Pan, M. Bray, and J. R. Bidlot, “Modelling of hurricane Dorian via the implementation of Wave Boundary Layer Model (WBLM) within the OpenIFS,” Ocean Model. (Oxf.), vol. 194, pp.102469, Apr. 2025. https://doi.org/10.1016/j.ocemod.2024.102469

[13] J. F. Townsend, G. Xu, Y. Jin, E. Yu, H. Wei, and Y. Han, “On the development of a generalized atmospheric boundary layer velocity profile for offshore engineering applications considering wind–wave interaction,” Ocean Eng., vol. 286, pp. 115621, Oct. 2023. https://doi.org/10.1016/j.oceaneng.2023.115621

[14] K. Ma, Y. Feng, J. Yang, Y. Cai, B. Yang, and X. Guan, “Two-stage distributionally robust optimization for port integrated energy system with berth allocation under uncertainties,” Energy, vol. 322, pp.135588, May 2025. https://doi.org/10.1016/j.energy.2025.135588

[15] Y. Wang, Y. Yang, L. Tang, W. Sun, and B. Li, “A Wasserstein based two-stage distributionally robust optimization model for optimal operation of CCHP micro-grid under uncertainties,” Int. J. Electr. Power Energy Syst., vol. 119, pp.105941, Jul. 2020. https://doi.org/10.1016/j.ijepes.2020.105941

[16] J. Wen, R. Jia, G. Cao, Y. Guo, Y. Jiao, W. Li, et al., “Multi-source shared operation optimization strategy for multi-virtual power plants based on distributionally robust chance constraint,” Energy, vol. 322, pp.135761, May 2025. https://doi.org/10.1016/j.energy.2025.135761

[17] A. S. Alghamdi, “A cloud-metaheuristic-based framework for stochastic optimization of a hybrid wind/hydrogen based-Fuel cell system in distribution network considering uncertainty,” Int. J. Electr. Power Energy Syst., vol. 169, pp.110778, Aug. 2025. https://doi.org/10.1016/j.ijepes.2025.110778